Lab 8
Binary Divider

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity binary_divider is

port

(

clk      : in  std_logic;

reset    : in  std_logic;

dividend : in  std_logic_vector(3 downto 0);

divisor  : in  std_logic_vector(3 downto 0);

start    : in  std_logic;

quotient : out std_logic_vector(3 downto 0);

remi     : out std_logic_vector(3 downto 0);

done     : out std_logic;  -- Added done signal

-- Debug outputs

debug_state : out integer range 0 to 6;

debug_counter : out integer range 0 to 4

);

end binary_divider;

architecture beh of binary_divider is

type state_type is (IDLE, LOAD, CHECK, SUBTRACT, SHIFT, DONE_STATE);

signal current_state, next_state : state_type;

signal ender        : std_logic_vector(7 downto 0);

signal quotient_reg : std_logic_vector(3 downto 0);  -- internal quotient

signal sor          : std_logic_vector(7 downto 0);

signal counter      : integer range 0 to 4;          -- Shift counter

begin

-- State register process - ONLY updates state

process(clk, reset)

begin

if reset = '1' then

current_state <= IDLE;

elsif rising_edge(clk) then

current_state <= next_state;

end if;

end process;

-- Debug output for state

with current_state select debug_state <=

0 when IDLE,

1 when LOAD,

2 when CHECK,

3 when SUBTRACT,

4 when SHIFT,

5 when DONE_STATE,

6 when others;

-- Debug output for counter

debug_counter <= counter;

-- Combinational logic for next state

process(current_state, start, counter)  -- Added counter to sensitivity list

begin

-- Default assignment

next_state <= current_state;

case current_state is

when IDLE =>

if start = '1' then

next_state <= LOAD;

end if;

when LOAD =>

next_state <= CHECK;

when CHECK =>

next_state <= SUBTRACT;

when SUBTRACT =>

next_state <= SHIFT;

when SHIFT =>

if counter = 3 then  -- Done after 4 iterations (0,1,2,3)

next_state <= DONE_STATE;

else

next_state <= CHECK;  -- Go back to CHECK for next iteration

end if;

when DONE_STATE =>

next_state <= IDLE;

end case;

end process;

-- Datapath process - ONLY place where ender, sor, quotient_reg, counter are assigned

process(clk, reset)

begin

if reset = '1' then

ender        <= (others => '0');

sor          <= (others => '0');

quotient_reg <= (others => '0');

counter      <= 0;

quotient     <= (others => '0');  -- Reset outputs

remi        <= (others => '0');

done         <= '0';

elsif rising_edge(clk) then

-- Default: done is low unless we're in DONE_STATE

done <= '0';

case current_state is

when IDLE =>

quotient_reg <= (others => '0');

counter      <= 0;

done         <= '0';

when LOAD =>

ender   <= "0000" & dividend;

sor     <= divisor & "0000";

counter <= 0;  -- Reset counter on load

when CHECK =>

null;  -- Just a pipeline stage

when SUBTRACT =>

-- Compare and conditionally subtract

if unsigned(ender) >= unsigned(sor) then

-- Remainder >= Divisor: subtract and set quotient bit to 1

ender        <= std_logic_vector(unsigned(ender) - unsigned(sor));

quotient_reg <= '1' & quotient_reg(3 downto 1);  -- Shift right, insert 1 on left

else

-- Remainder < Divisor: don't subtract, set quotient bit to 0

quotient_reg <= '0' & quotient_reg(3 downto 1);  -- Shift right, insert 0 on left

end if;

when SHIFT =>

-- Shift divisor right by 1

sor     <= '0' & sor(7 downto 1);

counter <= counter + 1;

when DONE_STATE =>

-- Reverse the quotient bits (bit 3 becomes bit 0, etc.)

remi(3) <= quotient_reg(0);

remi(2) <= quotient_reg(1);

remi(1) <= quotient_reg(2);

remi(0) <= quotient_reg(3);

quotient     <= ender(3 downto 0); -- Remainder

done     <= '1';               -- Signal completion

end case;

end if;

end process;

end beh;

In this I am implementing a binary divider and have attempted a division of 7/2 and result is also correct. Using internal quotient and Counter signals to detect the shift in the divisor. 


Lab 7
Booth’s Algo

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity booth is

generic(

N : integer := 5

);

port(

clk   : in  std_logic;

rst   : in  std_logic;

start : in  std_logic;

M     : in  signed(N-1 downto 0);  -- Multiplicand

Q_in  : in  signed(N-1 downto 0);  -- Multiplier

P     : out signed(2*N-1 downto 0); -- Final product

done  : out std_logic

);

end entity;

architecture Behavioral of booth is

-- Booth's Algorithm Registers

signal A     : signed(N-1 downto 0) := (others => '0');  -- Upper half

signal Q     : signed(N-1 downto 0) := (others => '0');  -- Lower half

signal Qm1   : std_logic := '0';                         -- Q(-1)

signal SC    : integer range 0 to N := 0;                -- Counter

signal M_reg : signed(N-1 downto 0) := (others => '0');  -- Stored multiplicand

-- State machine

type state_type is (IDLE, INIT, CHECK_BITS, ADD_SUB, SHIFT_REG, FINISH);

signal state : state_type := IDLE;

begin

process(clk)

begin

if rising_edge(clk) then

if rst = '1' then

A     <= (others => '0');

Q     <= (others => '0');

Qm1   <= '0';

SC    <= 0;

M_reg <= (others => '0');

done  <= '0';

state <= IDLE;

else

case state is

when IDLE =>

done <= '0';

if start = '1' then

state <= INIT;

end if;

when INIT =>

-- Initialize: A=0, Q=Multiplier, Qm1=0, SC=N

A     <= (others => '0');

Q     <= Q_in;

Qm1   <= '0';

M_reg <= M;

SC    <= N;

state <= CHECK_BITS;

when CHECK_BITS =>

-- Just move to ADD_SUB (checking happens there)

state <= ADD_SUB;

when ADD_SUB =>

-- Check Current Bit (Q0) and Previous Bit (Qm1)

if Q(0) = '0' and Qm1 = '1' then

-- 01: Add Multiplicand to A

A <= A + M_reg;

elsif Q(0) = '1' and Qm1 = '0' then

-- 10: Subtract Multiplicand from A

A <= A - M_reg;

end if;

-- 00 or 11: No operation

state <= SHIFT_REG;

when SHIFT_REG =>

-- Arithmetic Right Shift of [A, Q, Qm1]

-- This shifts the entire Product Register as one unit

Qm1 <= Q(0);                      -- Q(0) moves to Qm1

Q   <= A(0) & Q(N-1 downto 1);    -- Q shifts, gets A(0)

A   <= A(N-1) & A(N-1 downto 1);  -- A shifts with sign extension

-- Decrement counter

SC <= SC - 1;

-- Check if done

if SC = 1 then

state <= FINISH;

else

state <= CHECK_BITS;

end if;

when FINISH =>

done  <= '1';

state <= IDLE;

end case;

end if;

end if;

end process;

-- Final product is concatenation of A and Q

P <= A & Q;

end architecture;

This design implements Booth’s signed multiplication using an FSM, where the accumulator and multiplier registers are conditionally added or subtracted based on adjacent multiplier bits, followed by arithmetic right shifts repeated for N cycles to produce a 2N-bit product

Lab 9

--Two way lights

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity lights is port

(

stby          : in std_logic;  -- Standby mode (active high)

clk           : in std_logic;

light1_red    : out std_logic;

light1_yellow : out std_logic;

light1_green  : out std_logic;

light2_red    : out std_logic;

light2_yellow : out std_logic;

light2_green  : out std_logic

);

end lights;

architecture beh of lights is

-- State machine

type state_type is (YY, RG, RY, GR, YR);

signal state, next_state : state_type := YY;

signal counter : integer := 0;

-- Timing constants for 60 Hz clock

-- constant RG_TIME : integer := 1800;  -- 30 seconds

-- constant RY_TIME : integer := 300;   -- 5 seconds

-- constant GR_TIME : integer := 2700;  -- 45 seconds

-- constant YR_TIME : integer := 300;   -- 5 seconds

-- For simulation, use smaller values:

constant RG_TIME : integer := 30;

constant RY_TIME : integer := 5;

constant GR_TIME : integer := 45;

constant YR_TIME : integer := 5;

begin

---------------------------------------

-- Sequential Process

---------------------------------------

process(clk)

begin

if rising_edge(clk) then

state <= next_state;

-- Counter logic: reset on state change

if state /= next_state then

counter <= 0;

else

counter <= counter + 1;

end if;

end if;

end process;

---------------------------------------

-- Combinational Process

---------------------------------------

process(state, counter, stby)

begin

-- Default outputs (all off)

light1_red    <= '0';

light1_yellow <= '0';

light1_green  <= '0';

light2_red    <= '0';

light2_yellow <= '0';

light2_green  <= '0';

case state is

-- Standby state: Both Yellow (indefinite)

when YY =>

light1_yellow <= '1';

light2_yellow <= '1';

-- Exit standby when stby goes low

if stby = '0' then

next_state <= RG;

else

next_state <= YY;

end if;

-- RG state: Light1=Red, Light2=Green (30s)

when RG =>

light1_red   <= '1';

light2_green <= '1';

-- Check for standby mode first

if stby = '1' then

next_state <= YY;

elsif counter >= RG_TIME then

next_state <= RY;

else

next_state <= RG;

end if;

-- RY state: Light1=Red, Light2=Yellow (5s)

when RY =>

light1_red    <= '1';

light2_yellow <= '1';

if stby = '1' then

next_state <= YY;

elsif counter >= RY_TIME then

next_state <= GR;

else

next_state <= RY;

end if;

-- GR state: Light1=Green, Light2=Red (45s)

when GR =>

light1_green <= '1';

light2_red   <= '1';

if stby = '1' then

next_state <= YY;

elsif counter >= GR_TIME then

next_state <= YR;

else

next_state <= GR;

end if;

-- YR state: Light1=Yellow, Light2=Red (5s)

when YR =>

light1_yellow <= '1';

light2_red    <= '1';

if stby = '1' then

next_state <= YY;

elsif counter >= YR_TIME then

next_state <= RG;  -- Loop back to start

else

next_state <= YR;

end if;

when others =>

next_state <= YY;

end case;

end process;

end beh;

Simply just implemented the fsm and implemented the time duration using mathermatical operations.

Task 2
4-way chowk

--------------------------------------------------------------------------------

-- 4-WAY TRAFFIC LIGHT CONTROLLER - OPTION 2 (ADVANCED)

-- Each direction gets independent green light in rotation

-- Sequence: North → East → South → West → North (repeat)

--------------------------------------------------------------------------------

library ieee;

use ieee.std_logic_1164.all;

entity traffic_light_4way is port

(

clk    : in std_logic;   -- 60 Hz clock

reset  : in std_logic;   -- Active high reset

stby   : in std_logic;   -- Standby mode (all yellow)

-- North direction traffic light

north_red    : out std_logic;

north_yellow : out std_logic;

north_green  : out std_logic;

-- East direction traffic light

east_red     : out std_logic;

east_yellow  : out std_logic;

east_green   : out std_logic;

-- South direction traffic light

south_red    : out std_logic;

south_yellow : out std_logic;

south_green  : out std_logic;

-- West direction traffic light

west_red     : out std_logic;

west_yellow  : out std_logic;

west_green   : out std_logic

);

end traffic_light_4way;

architecture behavioral of traffic_light_4way is

-- Define 9 states

type state_type is (

STANDBY,

NORTH_GREENE,

NORTH_YELLOWE,

EAST_GREENE,

EAST_YELLOWE,

SOUTH_GREENE,

SOUTH_YELLOWE,

WEST_GREENE,

WEST_YELLOWE

);

signal current_state, next_state : state_type := STANDBY;

signal counter : integer := 0;

-- FOR SIMULATION: Use these small values

constant NORTH_GREEN_TIME : integer := 30;

constant EAST_GREEN_TIME  : integer := 25;

constant SOUTH_GREEN_TIME : integer := 30;

constant WEST_GREEN_TIME  : integer := 25;

constant YELLOW_TIME      : integer := 5;

-- FOR FPGA (60 Hz clock): Uncomment these and comment above

-- constant NORTH_GREEN_TIME : integer := 1800;  -- 30 seconds

-- constant EAST_GREEN_TIME  : integer := 1500;  -- 25 seconds

-- constant SOUTH_GREEN_TIME : integer := 1800;  -- 30 seconds

-- constant WEST_GREEN_TIME  : integer := 1500;  -- 25 seconds

-- constant YELLOW_TIME      : integer := 300;   -- 5 seconds

begin

----------------------------------------------------------------------

-- SEQUENTIAL PROCESS

----------------------------------------------------------------------

process(clk, reset)

begin

if reset = '1' then

current_state <= STANDBY;

counter <= 0;

elsif rising_edge(clk) then

current_state <= next_state;

if current_state /= next_state then

counter <= 0;

else

counter <= counter + 1;

end if;

end if;

end process;

----------------------------------------------------------------------

-- COMBINATIONAL PROCESS

----------------------------------------------------------------------

process(current_state, counter, stby)

begin

-- DEFAULT: All lights RED, yellows and greens OFF

-- These execute FIRST, before the case statement

north_red    <= '1';

north_yellow <= '0';

north_green  <= '0';

east_red     <= '1';

east_yellow  <= '0';

east_green   <= '0';

south_red    <= '1';

south_yellow <= '0';

south_green  <= '0';

west_red     <= '1';

west_yellow  <= '0';

west_green   <= '0';

-- NOW the case statement (overrides defaults)

case current_state is

when STANDBY =>

north_red    <= '0';

north_yellow <= '1';

east_red     <= '0';

east_yellow  <= '1';

south_red    <= '0';

south_yellow <= '1';

west_red     <= '0';

west_yellow  <= '1';

if stby = '0' then

next_state <= NORTH_GREENE;

else

next_state <= STANDBY;

end if;

when NORTH_GREENE =>

north_red   <= '0';

north_green <= '1';

if stby = '1' then

next_state <= STANDBY;

elsif counter >= NORTH_GREEN_TIME then

next_state <= NORTH_YELLOWE;

else

next_state <= NORTH_GREENE;

end if;

when NORTH_YELLOWE =>

north_red    <= '0';

north_yellow <= '1';

if stby = '1' then

next_state <= STANDBY;

elsif counter >= YELLOW_TIME then

next_state <= EAST_GREENE;

else

next_state <= NORTH_YELLOWE;

end if;

when EAST_GREENE =>

east_red   <= '0';

east_green <= '1';

if stby = '1' then

next_state <= STANDBY;

elsif counter >= EAST_GREEN_TIME then

next_state <= EAST_YELLOWE;

else

next_state <= EAST_GREENE;

end if;

when EAST_YELLOWE =>

east_red    <= '0';

east_yellow <= '1';

if stby = '1' then

next_state <= STANDBY;

elsif counter >= YELLOW_TIME then

next_state <= SOUTH_GREENE;

else

next_state <= EAST_YELLOWE;

end if;

when SOUTH_GREENE =>

south_red   <= '0';

south_green <= '1';

if stby = '1' then

next_state <= STANDBY;

elsif counter >= SOUTH_GREEN_TIME then

next_state <= SOUTH_YELLOWE;

else

next_state <= SOUTH_GREENE;

end if;

when SOUTH_YELLOWE =>

south_red    <= '0';

south_yellow <= '1';

if stby = '1' then

next_state <= STANDBY;

elsif counter >= YELLOW_TIME then

next_state <= WEST_GREENE;

else

next_state <= SOUTH_YELLOWE;

end if;

when WEST_GREENE =>

west_red   <= '0';

west_green <= '1';

if stby = '1' then

next_state <= STANDBY;

elsif counter >= WEST_GREEN_TIME then

next_state <= WEST_YELLOWE;

else

next_state <= WEST_GREENE;

end if;

when WEST_YELLOWE =>

west_red    <= '0';

west_yellow <= '1';

if stby = '1' then

next_state <= STANDBY;

elsif counter >= YELLOW_TIME then

next_state <= NORTH_GREENE;

else

next_state <= WEST_YELLOWE;

end if;

when others =>

next_state <= STANDBY;

end case;

end process;

end behavioral;

--------------------------------------------------------------------------------

-- OPERATION SUMMARY:

--------------------------------------------------------------------------------

-- Normal Cycle: NORTH_GREENE(30) → NORTH_YELLOWE(5) → EAST_GREENE(25) →

--               EAST_YELLOWE(5) → SOUTH_GREENE(30) → SOUTH_YELLOWE(5) →

--               WEST_GREENE(25) → WEST_YELLOWE(5) → back to NORTH_GREENE

--

-- Standby: Set stby='1' from any state to go to STANDBY (all yellow)

--          Set stby='0' to resume normal operation from NORTH_GREENE

--

-- Reset: Set reset='1' to immediately go to STANDBY state

--

-- For Simulation: Use the small timing constants (already set)

-- For FPGA: Uncomment the larger constants (1800, 1500, 300 cycles)

Lab 10

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

entity sram is

generic (

WIDTH : integer := 4;

DEPTH : integer := 4;

ADDR  : integer := 2

);

port (

Clock      : in  std_logic;

Enable     : in  std_logic;

ReadEn     : in  std_logic;

WriteEn    : in  std_logic;

ReadAddr   : in  std_logic_vector(ADDR-1 downto 0);

WriteAddr  : in  std_logic_vector(ADDR-1 downto 0);

Data_in    : in  std_logic_vector(WIDTH-1 downto 0);

Data_out   : out std_logic_vector(WIDTH-1 downto 0)

);

end sram;

architecture Behavioral of SRAM is

-- RAM array declaration

type ram_type is array (0 to DEPTH-1) of

std_logic_vector(WIDTH-1 downto 0);

signal tmp_ram : ram_type := (others => (others => '0'));

begin

--------------------------------------------------

-- READ PROCESS (Synchronous Read)

--------------------------------------------------

process (Clock)

begin

if rising_edge(Clock) then

if Enable = '1' then

if ReadEn = '1' then

Data_out <= tmp_ram(

to_integer(unsigned(ReadAddr))

);

else

Data_out <= (others => 'Z');

end if;

end if;

end if;

end process;

--------------------------------------------------

-- WRITE PROCESS (Synchronous Write)

--------------------------------------------------

process (Clock)

begin

if rising_edge(Clock) then

if Enable = '1' then

if WriteEn = '1' then

tmp_ram(

to_integer(unsigned(WriteAddr))

) <= Data_in;

end if;

end if;

end if;

end process;

end Behavioral;

Lab 11

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity Stack_FSM_Controller is

Port (

Clk      : in  STD_LOGIC; -- System Clock [cite: 76]

Rst      : in  STD_LOGIC; -- Active High Reset [cite: 76]

Push     : in  STD_LOGIC; -- Control input [cite: 77]

Pop      : in  STD_LOGIC; -- Control input [cite: 77]

Data_In  : in  STD_LOGIC_VECTOR(7 downto 0); -- 8-bit input [cite: 73, 79]

Data_Out : out STD_LOGIC_VECTOR(7 downto 0); -- 8-bit output [cite: 81]

Full     : out STD_LOGIC; -- Status flag [cite: 82]

Empty    : out STD_LOGIC  -- Status flag [cite: 82]

);

end Stack_FSM_Controller;

architecture Behavioral of Stack_FSM_Controller is

-- RAM Array 16x8 as specified in block diagram [cite: 60, 74]

type RAM_ARRAY is array (0 to 15) of STD_LOGIC_VECTOR(7 downto 0);

signal stack_mem : RAM_ARRAY := (others => (others => '0'));

-- Stack Pointer (SP) [cite: 17]

signal sp : integer range 0 to 16 := 0;

-- State Definitions [cite: 65, 66, 67, 68]

type state_type is (s_Idle, s_Push, s_Pop);

signal current_state, next_state : state_type;

begin

-- FSM State Transition [cite: 64]

process(Clk, Rst)

begin

if Rst = '1' then

current_state <= s_Idle;

elsif rising_edge(Clk) then

current_state <= next_state;

end if;

end process;

-- Next State Logic with Exception Handling [cite: 33, 67, 68]

process(current_state, Push, Pop, sp)

begin

next_state <= current_state;

case current_state is

when s_Idle =>

-- Push only if not full [cite: 25, 67]

if Push = '1' and sp < 16 then

next_state <= s_Push;

-- Pop only if not empty [cite: 31, 68]

elsif Pop = '1' and sp > 0 then

next_state <= s_Pop;

else

next_state <= s_Idle;

end if;

when s_Push =>

next_state <= s_Idle;

when s_Pop =>

next_state <= s_Idle;

when others =>

next_state <= s_Idle;

end case;

end process;

-- Datapath Logic: Memory Array & SP Management [cite: 42, 51]

process(Clk)

begin

if rising_edge(Clk) then

if Rst = '1' then

sp <= 0; -- SP resets to 0

Data_Out <= (others => '0');

else

case current_state is

when s_Push =>

stack_mem(sp) <= Data_In; -- Write to SP [cite: 24]

sp <= sp + 1;             -- Increment SP [cite: 24]

when s_Pop =>

Data_Out <= stack_mem(sp - 1); -- Read from top [cite: 30]

sp <= sp - 1;                  -- Decrement SP [cite: 30]

when others =>

null;

end case;

end if;

end if;

end process;

-- Status Flags logic [cite: 36, 39]

Full  <= '1' when sp = 16 else '0';

Empty <= '1' when sp = 0 else '0';

end Behavioral;

This lab focuses on designing a hardware-based Last-In-First-Out (LIFO) stack using VHDL. Unlike RAM, which uses random access, a stack restricts access to the "Top" via a Stack Pointer (SP) that manages addresses automatically. The system integrates a 16x8 RAM array with a Finite State Machine (FSM) to control PUSH and POP operations. Key objectives include handling memory exceptions like Overflow (stack full) and Underflow (stack empty) using dedicated status flags. Verification involves simulating the design to ensure data is retrieved in the correct reverse-chronological order

Lab 12
library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.NUMERIC_STD.ALL;

entity Cache_Controller is

Port (

Clk         : in  STD_LOGIC;

Reset       : in  STD_LOGIC;

Start_Btn   : in  STD_LOGIC;

Switch_Addr : in  STD_LOGIC_VECTOR(7 downto 0);

Led_Hit     : out STD_LOGIC;

Led_Miss    : out STD_LOGIC

);

end Cache_Controller;

architecture Behavioral of Cache_Controller is

-- 1. Define States [cite: 82]

type state_type is (s_IDLE, s_READ, s_COMPARE, s_RESULT);

signal current_state, next_state : state_type;

-- 2. Define Memory (Tag RAM 8x5) [cite: 43, 83]

type tag_array is array (0 to 7) of STD_LOGIC_VECTOR(4 downto 0);

constant TAG_MEMORY : tag_array := (

0 => "00001", -- Index 000: Tag 00001 (0x08) [cite: 68]

1 => "11111", -- Index 001: Tag 11111 (0xF9) [cite: 69]

7 => "10101", -- Index 111: Tag 10101 (0xAF) [cite: 70]

others => "00000" -- All other lines are empty [cite: 71]

);

-- Internal Signals

signal stored_tag : STD_LOGIC_VECTOR(4 downto 0);

signal input_tag  : STD_LOGIC_VECTOR(4 downto 0);

signal index      : integer range 0 to 7;

signal match      : STD_LOGIC;

begin

-- Extract Index and Tag from Switch_Addr [cite: 41, 42]

-- Index: [2:0], Tag: [7:3]

index     <= to_integer(unsigned(Switch_Addr(2 downto 0)));

input_tag <= Switch_Addr(7 downto 3);

-- 3. Process 1: Sequential State Transitions [cite: 84]

process(Clk, Reset)

begin

if Reset = '1' then

current_state <= s_IDLE;

elsif rising_edge(Clk) then

current_state <= next_state;

end if;

end process;

-- 4. Process 2: Combinational Logic [cite: 85]

process(current_state, Start_Btn, index, input_tag, stored_tag, match)

begin

-- Default values

next_state <= current_state;

Led_Hit    <= '0';

Led_Miss   <= '0';

case current_state is

when s_IDLE =>

if Start_Btn = '1' then

next_state <= s_READ;

else

next_state <= s_IDLE;

end if;

when s_READ =>

-- Retrieve Tag from array [cite: 87]

-- In simulation/FPGA, this state provides the clock cycle for memory access

next_state <= s_COMPARE;

when s_COMPARE =>

-- Perform comparison [cite: 87]

next_state <= s_RESULT;

when s_RESULT =>

-- Latch result to LEDs [cite: 88]

if (TAG_MEMORY(index) = input_tag) then

Led_Hit  <= '1';

Led_Miss <= '0';

else

Led_Hit  <= '0';

Led_Miss <= '1';

end if;

-- Stay here until button is released [cite: 37]

if Start_Btn = '1' then

next_state <= s_RESULT;

else

next_state <= s_IDLE;

end if;

end case;

end process;

end Behavioral;